Title

(SS3) for HA and HF Mill belt removal/installation

PURPOSE

Alternative safe work method for HA and HF Mill belt removal/installation with drive power on and belt tensioning pneumatics applied.

SCOPE

ZB Mill, removing and installing interior and exterior elevator belts

ZB HF Mills

| WHO: | NZB RTs |
| --- | --- |
| WHEN: | During Mill elevator belt replacement |
| WHERE: | ZB HA mills and HF mills |
| RISKS: | Movements of drive rollers, idle rollers, tension cylinders, and pinch points |
| COMMENTS: | Drive motor energized and turning. Air on for tracking |

OBJECTIVE OF THE SS3 TASK

Remove/Install Mill belts with Drive assistance and pneumatics for tracking/tension

ALTERNATIVE METHOD (SS3)

Removing the exterior and interior elevator belts - Notify operator - De-energize pneumatic elevator belt tensioning - Pull the belt cable - Attach pull piece - Pull while running the elevator belt drive in the proper direction. - Stay clear of rotating rollers and components - Stay clear of pinch points caused by the belt and or pulling rope (do not grasp the pulling rope!) - Wig-wag the belt into a pile or roll up and clear from the mill area - Repeat for interior belt removal - Perform the proper LOTOTO for any work that may take place after belt removal Installing the interior and exterior elevator belts - Attach the belt to the pull rope with the pull piece - Run the elevator belt drive in tandem with the folk lift or winch pulling the rope. - Stay clear of rotating rollers and components - Stay clear of pinch points caused by the belt and or pulling rope (do nope grasp the pulling rope!) - Insert the belt cable - Tension and track the belt - Repeat to exterior belt install - Reinstall all guarding and close access doors - Unlock remaining LOTOTO if needed - Re-energize machine and notify operator

| RTS Signature | EP Signiture | Shop Manager (Optional) |
| --- | --- | --- |
| Name Date | Name Date | Name Date |