| Application Date: | Supersedes: |
| --- | --- |
| Nature of revision: |  |

# PURPOSE: Facilitate mixer weight replacement

# SCOPE: instructions apply to farrel f270 pneumatic mixer pistons (non-keel weight)

# Definitions: hopper cover—square plate bolted to top of hopper that piston is bolted to.

# Instruction [Content]

  - Remove the Piston

  - Lock out mixer rotors, piston, hydraulics and relevant product feed devices.

  - Piston must be raised and pinned before lockout.

  - Remove or uncouple air and lube lines to the piston that will interfere with removal.

  - Remove check valve and piping from top of piston;

  - Using a bolt through a cross piece or a properly sized eye bolt, screw bolt into threaded hole in the top of the piston rod to secure the piston in the up position.

  - Remove the guards, covers, and aspiration ducts as necessary.

  - Remove 2 bolts form each side of the piston cover and bolt on the lifting plates.

  - Hook over head crane to lift plates using appropriate rigging.

  - Raise crane to point where cable is loaded to support piston while bolts are removed.

  - Remove the 4 lock wired bolts from the inside upper edge of the feed hopper. F370 mixers do not have these bolts.

  - Remove the 8 bolts from the top of the hopper cover.

  - Remove the piston.

  - Removal of weight

  - Take piston to Central Shop & place piston and weight in jig that is designed to hold it securely.

  - Remove cover plate to expose pin.

  - Remove pin with hydraulic puller.

  - Lift piston rod out of old weight.

  - Set piston rod into the new weight and bushing assembly.

  - Install new weight pin.

  - Secure cover plate.

  - Reinstallation of the piston.

  - Clean and lubricate the bolts.

  - Clean out threaded holes in the mixer hopper. Chase threads if needed.

  - Lower piston over hopper. Start bolts to locate hopper before touch down.

  - Torque the bolts to the following values using a torque wrench and multiplier

or a hydraulic torque wrench if available. Multipliers may be stacked if

necessary.

1 ¼ in. bolts – 1480 ft/lbs                   1 in. bolts – 740 ft/lbs

30 mm bolts – 1455 ft/lbs                   24mm bolts – 735 ft/lbs

  - Lock wire the bolts inside the mixer feed hopper.

  - Reinstall air and lube lines. It may be necessary to prime oil and grease lines to insure timely lubrication of the piston.

  - Clean up.

  - Check feed belts and under mixer for tools, trash or metallic debris.

  - Unlock rotors , piston, air, feed belts, and hydraulics , turn on disconnects and air.

  - Inform production of job completion and of the need to rub a cleaning rubber or nose cones to minimize contamination before beginning production.

- Reference Documents/Forms

| Farrel Print | 478-70074 for F270 |
| --- | --- |
| Farrel Print | 480BM70015 for F370 |