Title ZB Mill Folding Belt tracking SS3

PURPOSE

Alternative work method (ss3) for tracking ZB Mill folding belts

SCOPE

ZB Mill tracking of folding belts

ZB HF Mills

| WHO: | Tec RTs |
| --- | --- |
| WHEN: | After installation or re-tracking of a folding belt |
| WHERE: | ZB HF Mills |
| RISKS: | Rotating rollers, moving belts, translation drift, Pinch/Nip points, sharps, working on a ladder. |
| COMMENTS: |  |

OBJECTIVE OF THE SS3 TASK

Tracking of a folding belt

ALTERNATIVE METHOD (SS3)

- Notify operator - Obtain a proper ladder for access as needing (clear the floor area and inspect ladder) - *Clarify if you need to be in manual or MR mode for partial LOTOTO - Lock out the Folding Belt translation - Take note of sharps, nip and pinch points. Observe the area around and under you. - Note the translation may drift with disconnect turned off. - Run the folding belt rotation. Observe at a distance before approaching the belt. Belt edges and belt cables - Stay clear of rotating rollers and components - Loosen jam nut as needed - Adjust the tracking and observe - Tighten jam nut if applicable - Remove LOTOTO - Validate operation of translation and rotation - Return to production - Notify production

| Tech Representative Signature | EP Representative SIGNATURE | Shop Manager (Optional) |
| --- | --- | --- |
| Name Date | Name Date |  |